MINISTRY OF EDUCATION AND SCIENCE OF THE REPUBLIC OF

KAZAKHSTAN

“Kazakh-British Technical University” JSC

School of Mathematics and Cybernetics

ADMITTED TO DEFENCE

Head of School of Mathematics and Cybernetics,

Ph.D., Assistant Professor

___________ A. A. Issakhov

“___”___________ 2019

EXPLANATORY NOTE

TO GRADUATION PROJECT (work)

Theme: “Artificial Intelligence techniques based Adaptive Traffic Light Control”

Student

________________ A. Z. Karatay

“___”____________ 2019

Major: 5B070200

“Mathematical and Computer Modelling”

Supervisor

Professor,

Doctor of Math and Physics

________________ N. S. Dairbekov

“___”____________ 2019

Norms Compliance Monitor

Manager

____________ S. K. Abilda

“___”____________ 2019

Almaty, 2019

MINISTRY OF EDUCATION AND SCIENCE OF THE REPUBLIC OF KAZAKHSTAN

“Kazakh-British Technical University” JSС

School of Mathematics and Cybernetics

Specialty 5B070200 ““Mathematical and Computer Modelling”

ADMITTED TO DEFENCE

Head of School of Mathematics and Cybernetics,

Ph.D., Assistant Professor

___________ A. A. Issakhov

“___”___________ 2019

DIPLOMA PROJECT ASSIGNMENT

Student: Assiya Karatay

Project title: “Artificial Intelligence techniques based Adaptive Traffic Light Control”

Approved by the KBTU order: № _______dated “___” ___________

Submission deadline: ____________________

List of issues addressed in the diploma project or its brief content:

- Supervised learning

- Objective function’s optimization

- Problem statement and proposed solutions

- Experiments, implementation and improvement prospects

List of graphics (with precise indication of obligatory drawings):

- Tables (11).

- Figures (12).

Consultations regarding the project with indication of respective reasons:

| Section | Consultant (academic degree, title) | Timeline | Signature |
| --- | --- | --- | --- |
| Main | # N. S. Dairbekov # Professor, Doctor of Math and Physics | 14.01-15.03 |  |
| Special | D.B.Kurmankhojayev MSc of Computer Science | 16.03-6.05 |  |

DIPLOMA PROJECT WORK SCHEDULE

| Type of work | Submission deadline | Notes |
| --- | --- | --- |
| - Choosing the theme; - Drafting a work schedule for the implementation of the graduation project with specific deadlines. | January, 2019 |  |
| - Gathering information; - Review of the literature; - Review of the online sources. | January, 2019 |  |
| - Receiving and discussing the graduation work tasks; - Determining the purposes and tasks of the graduation work, statement of the problem; - Survey of the existing analogs of differential geometry on discrete spaces; - Attempt to devise a new analog of differential geometry on discrete spaces. | February, 2019 |  |
| - Working on the direct method of training on history; - Implementation of the direct method in Python; | March, 2019 |  |
| - Learning about supervised learning; - Working on the fully-connected neural network (FCNN) algorithm; - Implementation of the RFCNN algorithm in Python; - Preliminary defense of the graduation project. | April, 2019 |  |
| - Preparation of the explanatory note; - Getting feedback reviews. | May, 2019 |  |

# Project supervisor             __________________                         N. S. Dairbekov

(signature)

Student                              _____________________                   A. Z. Karatay

(signature)

Date  «____» ________________2019

# Project sheet

| № | Format | Designation | Denomination | Quantity | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | А4 |  | Explanatory note | 1 | 68p |
| 2 | .pdf |  | Electronic copy of the diploma project | 1 |  |
| 3 | .pptx |  | Presentation Files | 1 |  |
| 4 | zip |  | Python source files | 1 |  |

# ABSTRACT

The explanatory note contains 68 pages of typewritten text, 2 tables, 19 figures, the list of literature – 5 references and 1 appendix.

TRAFFIC SIGNAL CONTROLLER, TRANSPORTATION SYSTEM, MACHINE LEARNING, SUPERVISED LEARNING, OBJECTIVE FUNCTION.

The objective of the current research project is to build an adaptive traffic signal controller for isolated two intersections. The problem is formulated as a supervised learning task. The proposed solutions are based on a fully-connected neural network. In contract with other studies in the field, we use both queue length and the average delay as a measure of performance. A state is represented by the number of queuing vehicles for each intersection. In this work, we propose two different supervisors for neural networks to determine which is better. We give full theoretical explanation and conduct an experiment in the simulated environment for each of the proposed solution.

АҢДАТПА

Дипломдық жоба машинамен басылған 68 беттен, 2 кестеден, 19 суреттен, 5 пайдаланылған әдебиеттен және 1 қосымшадан тұрады.

БАҒДАРШАМ БАСҚАРУШЫСЫ, ТРАНСПОРТТЫҚ ЖҮЙЕ, МАШИНАЛЫҚ ОҚЫТУ, НҰСҚАУШЫМЕН ОҚЫТУ, МАҚСАТТЫ ФУНКЦИЯ.

Осы зерттеу жұмысының мақсаты жекеленген екі қиылысқа адаптивті бағдаршам басқарушысын жасау болып табылады. Тапсырма нұсқаушымен үйрену тапсырмасы ретінде қойылған. Ұсынылған шешеім әдістері нейрондық желіге негізделген. Осы саладағы басқа зерттеулерден ерекшелігі алгоритмнің орындалу көрсеткіші ретінде орташа күту уақыты мен тұрақты машиналардың кезегінің ұзындығы қатар қолданылады. Қиылыстың жағдайы ретінде қиылыс алдында тұрған көліктердің саны алынады. Біз, сондай-ақ, осы жұмыста қай нұсқаушы жақсырақ үйрететінін  анықтау үшін нейрондық жүйені оқытуға екі нұсқаушыны аламыз. Біз ұсынылған шешімдердің әрқайсысына толық теориялық негіздемені келтіреміз және эксперименттер жүргіземіз.

АННОТАЦИЯ

Дипломная работа содержит 68 страниц машинописного текста, 2 таблицыб 19 рисунковб список использованных источников – 5 наименований и 1 приложение.

КОНТРОЛЛЕР СВЕТОФОРА,  ТРАНСПОРТНАЯ СИСТЕМА, МАШИННОЕ ОБУЧЕНИЕ, ОБУЧЕНИЕ С УЧИТЕЛЕМ, ЦЕЛЕВАЯ ФУНКЦИЯ.

Целью данной исследовательской работыявляется создание адаптивного контроллера светофора для изолированных двух перекрестков. Задача поставлена как задача обучения с учителем. Предложенные методы решения основаны на нейронной сети. В отличие от других исследований в данной области, в качсетве показателя производительности алгоритма мы используем длину очереди стоящих машин на ряду со средним временем ожидания. Состояние каждого перекрестка представлено количеством машин, стоящих перед перекрестком. Также в данной работе мы предлагаем сравнить два учителя для нейронных сетей для определения наилучшего из двух вариантов. Мы приводим полное теоретическое обоснование и проводим эксперименты для каждого из предлагаемых алгоритмов.

CONSENT

INTRODUCTION	12

1 TRANSPORTATION SYSTEM	14

1.1 Transportation system	15

1.2 Elements of supply. Traffic network structure	15

1.3 Signal timing and design	15

1.3 Traffic streams: macroscopic and microscopic parameters	15

1.4 Progressive systems	15

1.5 Elements of demand	15

1.6 Traffic Assignment: connecting supply and demand	15

2 GETTING DATA AND SIMULATION	18

2.1 Data collection for demand and volume estimation	19

2.2 Modelling and simulation of advanced decision making	21

3 ARTIFICIAL INTELLIGENCE	32

3.1 Artificial intelligence	32

3.2 Machine learning	32

3.3 Neural Networks	35

3.4 Anatomy of NN	36

4 PROBLEM STATEMENT AND PROPOSED SOLUTIONS	40

4.1 Problem statement	38

4.2 State space	52

4.3 Action space	53

4.4 Ground truth	58

4.5 Temporal-difference learning	45

4.6 Fully connected neural network	50

5    EXPERIMENTS, IMPLEMENTATION NOTES AND IMPROVEMENT PROSPECTS	59

5.1 Independent traffic signal control of two intersections	60

5.2 Joint traffic signal control of two intersections	63

5.3 Construction of objective function to find the best signal plan	65

CONCLUSION	72

REFERENCES	73

APPENDIX A

APPENDIX B

APPENDIX C

APPENDIX D

INTRODUCTION

1 TRANSPORTATION SYSTEM

In this chapter we discuss the basic concepts connected to the infrastructure of traffic and transportation system, which is considered as a supply side. We begin with definitions of network structure. After, describe traffic control measures.

  - TRANSPORTATION SYSTEM

It cannot be obvious how to define what is a transportation system and what are its components. We can find different definitions based on aim, context and perspective they are said about. More specifically, when we consider the modes of transportation, each kind of them can be seen as an individual system. Hence, a transportation system can be supposed to constitute by supply and demand parts. A supply part is engineering completion of problem as infrastructure providing capacity. A transportation system has a function to provide movement for all moveable things and creatures such as persons, animals, and plants. In this work, we consider a transportation system of persons.

A transportation system has various components such as road users (drivers, pedestrians, cyclists, motorcyclists); vehicles (both private and commercial), buses, and other transit means; streets and highways; traffic control and conventions, laws, enforcement policies, lightning, etc.). While we can easily see the clear relation of road users to the demand, vehicles and transit provides serving to this demand. Streets, highways, and control devices has a connection to supply.

To deal with a transportation system cannot be so easy, since there are at least three reasons why it is expected to be complex. Firstly, a system has indefinitely related units. Secondly, even the components are predictable, its emergent behavior is difficult to predict. Thirdly, small alteration in input variables can cause large changes in whole behavior. The last one demonstrates the dynamical quality of a system.

Additionally, road users’ behaviors are heterogeneous, so it impacts to the difficulty to predict traffic streams. This observation is significant in our problem, since in most cases researchers consider traffic flow based on fluid dynamics. Both being multiple particle systems, a flow of water through pipes can be predicted, whereas this is impossible in situation with road users.

  - Elements of supply. Traffic network structure

Networks are convenient to describe transportation systems from physical feature’s viewpoint. They are composed by nodes (vertices) and links (edges). Nodes illustrate stations, junctions, etc. There is mostly more than one way to travel between two given nodes. Links demonstrate roads, highways, rail lines, etc. Each link plays a role as platform for flow in one or two directions and has a distinct nominal capacity.

The subproblem of traffic flow description includes the measurement and analysis of parameters that can be categorized into two groups. They are macroscopic (volume, speed, and density of traffic streams) and microscopic (speed of individual vehicles, headway and spacing). Before going deeply into parameter, we should recall importance of roadways on the overall behavior of parameters.

Roadways with uninterrupted traffic flow represent no interruption such as intersections with traffic signals, stop or yield signs, etc. This can be seen in freeways bypassing residential areas. The characteristics of the traffic streams alter related to the interaction among vehicles. Weather conditions also has an effect on flow parameters, so the environment can cause complexity of a system. As a result, interrupted flows are more complex to model for the reason that movements are periodically constrained, for example, by a red signal.

1.3 Traffic streams: macroscopic and microscopic parameters

As already stated, traffic stream description involves a series of parameters that can be either macroscopic or microscopic. Speed (υ) is a microscopic property of each vehicle. Hence, an average speed can be associated with the whole stream. Average or mean speed is mostly calculated over all vehicles occupying a given section or passing a given point over some time period [Roess et al., 2004]. They are known as space-mean and time-mean respectively.

The most characteristic macroscopic parameters are the traffic volume (traffic flow, flow rate) q, and the density k. The traffic volume is the number of vehicles passing a point, a lane or direction of a road during a specified time interval like annual, daily, hourly amount of vehicles.

The density is the amount of vehicles in a given length of a road or lane. The relationship of density with speed comes from the fact that drivers choose speed policy depending how close vehicles are. Furthermore, density is an important feature of the traffic flow quality. Whereas it is difficult to measure density, modern detectors can measure occupancy, which is the proportion of time that a vehicle covers the detector in a defined time period. Otherwise, we can compute density as a function of flow and speed.

Besides speed, flow and density are also related to individual vehicles’ parameters. For example, average density refers to spacing, which means the distance between two successive vehicles. Headway is found as the time interval between successive vehicles, measured at a reference point, and is directly related to average flow.

The obtainability of macroscopic parameters from microscopic ones is obvious locally, however, the reverse operation is difficult.

Traffic control measures

The diversity of traffic control policies has not provided safe and efficient states to the road users yet. In this text, we concentrate our attention to traffic signal controllers distributing simple messages to the road users. This is called a signalized (regularized) intersection.

Traffic signals have different ways of realization ranging from hard-wired logic to computerized control. Their controllers are connected to display devices, generally, to lights. Controllers inform the road users when they may continue their movements, and when they must stop.

Traffic signal controllers use both hardware and software in order to implement a signal timing indications continuously, consistently, and efficiently. One important issue is that no conflicting movements receive green indication at the same time.

There are some advantages of using traffic signal control such as providing ordered traffic movement, reducing collisions and interrupting heavy traffic to allow other road users as pedestrians to cross. However, poor designation of controllers can decrease profitability of usage. On the other hand, we encounter disadvantages like excessive delays, disobedience of the signal indications, and use of less adequate alternatives.

Traffic signals can operate in various ways depending on dimensions. The first dimension of operation relates to the current traffic state. It has two modes as pretimed and actuated. In pretimed operation, a sequence of signal indications is fixed, otherwise, it may change on a periodic basis. In order to work in the last mode, controllers must be connected to detectors that provide full data about traffic stream.

The second dimension defines the coordination of traffic signal controllers. After synchronization our task is to provide a progressive movement.

1.3 Signal timing and design

According to Roess et al. [1], the most basic unit in signal design and timing is the cycle. This is defined as a complete rotation through all green indications. All legal vehicular movements (as for example in Figure 1) commonly receive a green indication during each cycle. Once a phasing plan is established considering left turns or not, the next step is to assign adequate timings for each signal phase to cope with demand totally. The main concept is to split the portion of green indication that each phase receives. The cycle length is the total time to complete one sequence of all movements around an intersection.

Figure 1: Signal plan with three phases.

Typically, cycle lengths differ in the range 60-150 s. However, there can be large deviations from these numbers. After the determination of cycle length, the effective green time is divided among the phases, proportionally to the critical lane volumes. These computations relate to a pretimed signal controller. The cycle length, phase sequence, and timing of each interval are constant and follow a predefined plan. The implementation of various signal plans at different times of the day or days of the week does not mean that this is actuated, since it is predefined.

It is another case when we come to actuated control, as this uses information about the current demand in order to make modifications in the intervals and phase sequence. We get that information from some detectors. Generally, an actuated controller is programmed to change the green time indication for each phase, so that the current demand is served the best way possible.

The advantage of an actuated signal timing varying on a cycle-by-cycle basis is important. For instance, if a pretimed operation is designed to serve 30 vehicles per cycle in a given phase, but in a given cycle only 10 arrive, as the controllers understand the detecting device signal as no demand, it alters the switches and interrupts the current phase. Conversely, if the demand is above 30 vehicles per cycle, the controller extends the cycle length. To avoid undesirable issues, actuated control must observe the following moments: a minimum and a maximum green time indication per phase; a way to define what happens in case of no demand; fixed time for yellow and all-red intervals; adaptation in the pedestrian intervals; and time to allow vehicles to travel from the detector to the stop line.

There are three kinds of actuated control. The full actuated operation requires that all lanes have detectors. At this type the entire phasing can be changed simultaneously with the phase sequence. The semi-actuated type is accommodated where a major arterial street intersects a small local street, in which detectors are installed. It receives green indication only in presence of a vehicle. The interruption can be programmed in a way to cause less disruption in the flow of the major arterial one. The third type of actuated control is similar to the full one but with minimum green phase duration, provided detectors capable of assessing the number of queued vehicles.

The disadvantages of actuated control are basically of two natures. Firstly, there are additional costs for installation and maintenance of detectors. The second factor appears if progressive systems in use. Since this kind of system requires all signals in the progression to operate with equal cycle length, these cannot differ on a cycle-by-cycle basis.

1.4 Progressive systems

Progressive systems are also called synchronized, coordinated systems. The purpose of progressive systems is to synchronize traffic signals along an arterial to provide travelling for vehicles without stopping at red lights. Thus, if appropriate signal plans are selected to run at adjacent traffic signals, a “green wave” is built, so that vehicles do not have to stop at intersections. The simplest way to implement a progressive system is to calculate synchronized signal plans for fixed times of the day such as morning and afternoon peak hours.

Progression imposes some restrictions on the phasing and timing of the signals. It was already mentioned that cycle length must be equal. Additionally, the beginning of green occurs at the same time relatively to the beginning of the green indication at the upstream and downstream indications. Exceptions are made in which the cycle length is a multiple of the basic lengths. Mostly, the cycle length is less than double of the basic lengths.

The other restriction comes from the so-called offset and can be better explained by means of a time-pace diagram, which is a plot of the traffic signal indications for two or more intersections along time. Such plots convey the conception of vehicles’ trajectories as a function of time. Figure 2 demonstrates this with six intersections in an arterial. Ideal progression is assumed to provide simplicity. Also let us assume that at time t=0, the signal turns green at position s=0. Vehicles there start or continue to move, arriving at the sixth intersection at t=140 and s=2000. This car and all those entering the arterial at the first intersection from time 0-70 will be able to pass the whole arterial without stopping. The time interval between the start of the green time at two adjacent intersections is called the offset and depends not only on the distance between these intersections, but also on the average speed of the progression, which is normally indicated in a sign. The offset can be determined on the other was as, for example, the distance relative to the initial intersection, but they are all variations of the basic concept just described.

Figure 2 also illustrates that there is a time window within moving vehicles through the series of intersections as signals turn green in order. This window is called the bandwidth of the progression and is determined by the split of the cycle length, but it may be smaller if conditions are not favorable. Trajectory lines in Figure 2 show the speed of progression. Of course, if vehicles deviate from the recommended speed, this causes either a narrowing of the bandwidth or the increase of stops.

Figure 2: Time-space diagram of a progression in an arterial: equal bandwidths of 70s.

Also, this progression is a particular, simple, case in which the bandwidth is designed to use all the green time in one direction. We should assume that this direction has a much higher traffic volume. Thus, one may expect vehicles traveling in the other direction to have to stop in some or all intersections. In fact, we can see that the progression in this example is efficient only for vehicles traveling northbound.

The classical problem concerning synchronization systems is to find the optimal bandwidth for different cycle times and speeds. Additionally, in fact, designing efficient offsets in two-way arterials is a classical optimization problem. At this work, we leave all these questions open and narrow the problem excluding that moments.

Relating the efficiency of progressive systems, it can be obtained in different levels of abstraction. Commonly one looks for optimization a weighted combination of stops and delays, a measure of the density (vehicles per unit of length) in the arterial or network, or travel time. These parameters are acceptable for under-saturated arterials or networks. In case of over-saturated networks, queues should be taken into account. When the demand excesses the capacity, the maximization of the throughput should be the main objective.

There are restrictions regarding the responsiveness of a traffic signal control. It is not an instant and frequent action to change the cycle length and the offset. Also the transition of information may take some time. In general, it is not feasible to implement many different coordination patterns within a short period of time. Therefore, careful and efficient real-time adjustments are questions of interest.

To sum up, synchronized systems are valuable in some situations. However, the methodology for creating a signal-timing makes a lot of simplifying assumptions, including a number of vehicles that reflects a “typical” condition. We know the behavior of traffic flow in the morning and afternoon peak hours, but it is unpredictable in other cases. So, signal timing may not be effective. Also, the location plays the role. The “secondary” streets become as important in business districts as arterial streets because of saturation of the last ones. Accidents and the weather conditions as snow and floods also impacts traffic patterns. Besides the already mentioned issue of cost and maintenance, as well as limitations imposed in the design of the signal timings, a further issue is that it is difficult to optimize several intersecting arterials simultaneously using real-time data and in congestion regimes. These issues show that simple off-line or on-line optimization of the synchronization in arterials may not be able to deal with changing traffic flow. Thus, more flexible approaches are necessary.

1.5 Elements of demand

At this chapter we look at basic concepts of demand as a counterpart to the transportation infrastructure. We already know that the infrastructure is fixed at some level, however the behavior of demand alters frequently because of users and their relevant choices. The end of traveling cannot be considered for the reason that people perform activities at various geographic locations. Hence, demand is highly differentiated by time of the day, day of the week, purpose, etc.

Traditionally, demand is represented in so-called “origin-destination-matrix” (OD matrix). Each entry in the OD matrix demonstrates the amount of users who want to travel from an origin location to a destination location. Figure 3 illustrates one example how OD table can be constructed from several numbered zones.

Figure 3: Basic concepts of an OD matrix as a representation of aggregated travel demand (adapted from Barcelo [2010], p.7).

1.6 Traffic Assignment: connecting supply and demand

It is necessary to define which parts of infrastructure are going to be used by travelers to predict the network flow. This leads to that demand model needs to be assigned to the available supply. Hence, the so-called traffic assignment (route or trip assignment) designs the interaction between the traffic system and driver’s decision making. The main challenge in modelling and controlling transportation systems is the difficulty that a system manager cannot control drivers in election of routes.

In the classical four-stage model (Figure 4), traffic assignment is the last phase. Its output represents the state of a transportation system, which is a relevant input for evaluating the consequences of the infrastructure alterations. Furthermore, for microscopic traffic flow simulation, data about routes chosen is relevant, either indirectly for determining turn probabilities at intersections, or directly for demonstrating the decision of a simulated road user at an intersection. This information is generated by the process of traffic assignment.

Figure 4: Schematic overview of the classical four-stage model of transportation modelling (adapted from Ortuzar and Willumsen [2001], p.23)

Generally, people choose routes for connecting their personal origins with their destinations. In a traditional aggregated and static view, the trips in the OD matrix are assigned to links that are part of a route from an origin to a destination. The costs of the single edges forming the route affect user decisions and demand flows. That costs are based on other travelers’ decisions about the routes they are going to use.

There is a plenty of approaches for trip assignment modelling. Cascetta [2009] provides a fundamental classification along the dimensions as interaction, supply and demand factors.

The main dimension is about the interaction between demand and supply. The selection of a traveler has an impact on the traffic load and hence on the availability of a particular path for the other users. A valuable method is composed by user equilibrium approaches in which the trip distribution relates the costs they produce.

The second dimension of supply factors is divided into two groups: whether the supply provides continuous services such as roads for vehicles, or only on a scheduled basis, such as public transportation. An essential dimension is whether there are assumptions on the feedback loop between load and travel-time, i.e., whether assignment is done in a un-congested network or a congested one.

There are four dimensions to differ in demand factors category: whether there is only one class of users, or multiple classes; whether demand is fixed, or variable in reaction to load; whether route choice is done completely pre-trip and does not alter during evaluation, or whether route choice is done pre-trip as well as en-route; and finally, whether there is a deterministic or a stochastic route choice.

- GETTING DATA AND SIMULATION

The main objectives of information collection in traffic engineering are to gather data to predict future transportation necessities for planning aims; to forecast effects of measures taken to develop efficiency; and to improve the understanding about parameters of the traffic flow model. These aims will be discussed in this chapter.

2.1 Data collection for demand and volume estimation

To estimate demand and volume we need commonly the amount of vehicle, passengers, etc. The number can be obtained by different devices and methods. Then from these countings some quantities as volume, flow, demand, and capacity can be obtained.

Demand is the number of vehicles or passengers that desire to travel. In some situations, demand can be higher than the volume. Capacity is a parameter of the facility itself.

While volume and flow can be counted, demand is more difficult to estimate, as it requires knowledge about the number of road users who have changed their planned routes. The major arterial way’s demand is more complex to predict than the freeway’s demand because of the different diversion intersections as an alternative.

To further complicate the problem, a characteristic pattern of traffic volume is its concentration on certain hours of the day and certain days of the week. Strong congestion may occur, for example, when people go to and back from workplaces. According to Roess et al. [2004], depending upon region and location, the peak hour typically contains 10-15% of the 24-hour volume.

Provided this parameter of seasonality and within-day variation, a question remains: what should be measured. Volumes can be measured for various time periods, for instance, 1-5 min. However, it seems to be statistically important to measure each 15 minutes. At the same way there are variations in daily patterns, as well as in patterns of road used for different aims.

At this point we would like to mention that concentration of traffic in well-known patterns is less and less a typical phenomenon in modern societies. A “typical” morning peak hours in mega-cities look more or less at this way: after 10 am, the traffic remains relatively high, showing that afternoon peaks no longer exist. This is a phenomenon caused by two factors. They are users making travel choices to avoid peaks and changes in the business nature and locations that are more spread. Not differentiating peaks, volumes need to be measured anyway. Volumes counted can be presented as in Figure 5, which depicts manual countings for an intersection of two arterials in the city of Porto Alegre (Brazil), collected (over 15 min and extrapolated to a period of 1 h) on June 6, 2013 from 08:26 to 08:41 am.

Figure 5: Example of vehicle countings for an intersection in Porto Alegre (AM peak).

To count the volume of vehicles or passengers that pass a given point, different methods and devices can be used, which fluctuate frequently in their complexity and cost. The simplest way is to use manual counts. Another way of count devices is those that are portable such as pneumatic road tubes.

Another measure that is mostly used is the occupancy of a detector: the fraction of the aggregation interval during which the cross section of the road is occupied by a vehicle.

Generally, studies referred to demand require information from OD matrices. Such data commonly includes surveys and other kinds of interviews in order to establish some travel patterns. Interviews could be made at home, by phone, or by stopping the vehicle and asking the travelers a short sequence of questions relating the origin, destination, aim, route, and eventually, modes involved in that trip. Alternatively, interviews could be carried out at centers that attract trips (shopping centers, etc.) in order to reduce scope of the questionnaire.

2.2 Modelling and simulation of advanced decision making

Modelling and simulation is one of the major application areas for computer science in the transportation area.

A great variety of existing traffic simulators have illustrated their value in the practice of traffic management and operations. Representing traffic systems in a model is an established means of forecasting the effects of a particular political or infrastructural measure. Simulation is also used to test the feasibility of technical innovations such as new communication or localization systems. In driving simulators, traffic simulation is used to generate the traffic context for the driving task.

Traffic model have existed for many years. Generally, their origins can be found in physics, mathematics, and statistics. During the last 20 years, various techniques from AI and computer science in common have figured out their way into traffic simulation. Agent-based simulation has become a relevant basic for a plenty of approaches to traffic simulation. This means that the decision making entities as travelers or households actively decide themselves about their interactions based on local data. Particularly, it is now possible to simulate each individual deciding locally about his departure time, about transportation mode, route, etc. In these decision making processes, models from cognitive intelligence science can be used.

A lot of various approaches to traffic simulation exists, with different levels of granularity reproducing the intelligent behavior of drivers to a greater or less extent.

3 ARTIFICIAL INTELLIGENCE

In this chapter we will describe the basic concepts and definitions of supervised learning which are essential for understanding the proposed solutions. In doing so, we follow the book “Deep learning with Python” by Francois Chollet [1].

3.1 Artificial intelligence

Artificial Intelligence (AI) has been one of the most popular themes in Computer Science for the last few decades. There has been written a myriad of articles providing convincing dreams about virtual assistants and self-driving cars. Having so intense hype, it is still a question for most people if the AIs are for real.

In the middle of the 20th century, a question if computers can “think” has raised. Thereby, a probable definition for artificial intelligence is the effort to automate intellectual tasks generally performed by human-being. [6] Thus, AI is a common area which covers both machine learning and deep learning, and other approaches that do not include any learning (Figure 6).

Figure 6. Artificial Intelligence, Machine Learning and Deep learning

In the beginning of this science, there was high demand hard-coded programming with proficient logic. So, expert programmers played the major role in handling many problems, for example, playing chess. The faster time goes, the more complex tasks were found as text, speech, and face recognition, which influenced the growth of machine learning.

3.2 Machine learning

In comparison with classical programming, which gets data and rules as input and gives out answers, machine learning works differently (Figure 7). With ML, a programmer put data and answers expected from the data, and get as a result rules that can be applied to new data to make predictions. This is actually one part of Machine Learning called Supervised Learning.

Figure 7: Machine learning: a new programming paradigm

There are several various types of learning algorithms. The main three types are supervised learning, unsupervised learning and reinforcement learning.

Supervised learning is the most frequently studied kind of learning. In supervised learning we explicitly teach the computer on a set of labeled examples provided by an educated external supervisor, knowing that this data represents completely correct past behavior. The objective for this kind of learning is for the system to extrapolate, or generalize, its responses so that it acts correctly in situations not present in the training set. For instance, let us suppose we are given a house selling database in which each record is represented by a set of features of the house (e.g. size, number of bedrooms, etc.) and the label with a price the house has been sold for. Given this database as a training set, the supervised learning algorithm will be able to predict the most appropriate price for a new house with its features close, but not exactly equal to those contained in the training database. This is an example of regression problem in which the learning algorithm outputs a continuous value. There is also a classification problem in which the algorithm, given a set of features characterizing the object belongs to. For example, let us say we have a dataset of medical records in which each entry is specified by the breast tumor size and is labeled to be either malignant or benign. Also, let us suppose we have a new patient who tragically has a breast tumor, and by looking at the size of the tumor we would like to determine whether she has a cancer or not. By learning on the dataset the algorithm will be able to classify this particular tumor to be either malignant or benign to a certain extent of accuracy.

As for unsupervised learning, roughly speaking, it is when we let the computer learn by itself. In other words, unsupervised learning is concerned with finding structure hidden in collections of unlabeled data. Given a dataset of objects that have no labels at all, the unsupervised machine learning algorithm may break these data into separate clusters – groups of objects that are in some sense similar to one another. This is called a clustering algorithm. One example where clustering is used is in Google News where the search engine automatically groups into one cluster al the websites on the web that are related to a particular event. A user interested in reading about the event is free to choose any website suggested by the engine. Another examples might be processing astronomical data or identifying cohesive groups of friend based on their activity in a social network.

Reinforcement learning falls into neither of the two above-mentioned categories of learning. It is not a supervised learning, since we do not have a predefined exemplary dataset of correct behavior of an agent within the environment. However, a solution method which uses a history of the past behavior of the agent in order to train the policy function might be viewed as supervised learning to some extent. Nevertheless, since in general reinforcement learning problems are interactive in nature, it is often impractical to obtain examples of desired behavior that are both correct and representative of all the situations in which the agent has to act. Reinforcement learning is also different from unsupervised learning, for the goal of reinforcement learning is not to find a hidden structure in the agent’s behavior, but rather to maximize the reward signal. Revealing structure is an agent’s experience might certainly be useful, yet by itself it does not address the issue of maximizing a reward signal. Thus, reinforcement learning is often considered to be the third machine learning paradigm.

3.3 Neural Networks

The neural network can approximate any continuous function. One way to look at Neural Networks with fully-connected layers is that they define a family of functions that are parameterized by the weights of the network. Neural networks and more complex models are able to achieve much better accuracy than linear models because they no longer have the restriction of just learning a single template per category.

We know the general linear score function:

f = W  * x

The score function can also be represented by, for instance, a 2-layer NN:

f = W2 * max(0, W1 * x)

It turns out that Neural Networks with at least one hidden layer are universal approximators. Given any continuous function f(x) and some ɛ > 0, there exists a Neural Network g(x) with one hidden layer (with a reasonable choice of non-linearity, e.g. sigmoid) such that

For all x; |f(x) - g(x)| < ɛ.

If one hidden layer success to approximate any function, why use more layers and go deeper? The fact that deeper networks (with multiple hidden layers) can work better than a single hidden-layer networks is an empirical observation, despite the fact that their representational power is equal.

In practice it is often the case that 3-layer neural networks will outperform 2-layer nets, but going even deeper (4,5,6-layer) rarely helps much more. However, the depth in Convolutional Networks has been found to be an extremely important component for a good recognition system (e.g. on order of 10 learnable layers). One argument for this observation is that images contain hierarchical structure (e.g. faces are made up of eyes, which are made up of edges, etc.), so several layers of processing make intuitive sense for this data domain.

The whole story in deep learning is coming up with the right structure for the score function f. You can imagine writing down different functional forms for how to combine weights and data in different complex ways.

A deep learning model is simply a directed acyclic graph of layers. The most common instance would be a linear stack of layers, mapping a single input to a single output. However, as you move forward, you will be exposed to a much broader variety of network topologies. Some common ones include:

- Two-branch networks

- Multi-head networks

- Inception blocks

Picking the right network architecture is more an art than a science, and while there are some best practices and principles you can rely on, only practice can really help you become a proper neural network architect (Deep Learning with Python, Francois Chollet).

3.4 Anatomy of NN

The core building block of neural networks is the layer, a data-processing module which you can conceive as a “filter" for data. Some data comes in, and comes out in a more useful form. Precisely, layers extract representations out of the data fed into them. Most of deep learning really consists of chaining together simple layers which will implement a form of progressive "data distillation".

Each layer consists of number of neurons (Figure 7). The neuron is nothing more than a set of inputs, a set of weights, and an activation function. The neuron translates these inputs into a single output, which can then be picked up as input for another layer of neurons later on. Non-linear behavior in a neural network is accomplished by use of an activation function.

Figure 7: A neuron of ordinary neural network.

To make our network ready for training, we need to pick three more things:

- Loss function: it is how the network will be able to measure how good a job it is doing on its training data, and thus how it will be able to steer itself in the right direction.

- Optimizer: this is the mechanism through which the network will update itself based on the data it sees and its loss function.

- Metrics to monitor during training and testing

In general, all machine learning systems in our time use tensors as their basic data structure. A tensor is a container for data|almost always numerical data. They are a generalization of matrices to an arbitrary number of dimensions.

Learning simply means a _nding a combination of model parameters that minimizes a loss function for a given set of training data samples and their corresponding targets. This is done by drawing random batches of data samples and their targets, and computing the gradient of the score function parameters with respect to the loss on the batch. The score function parameters are then moved "a bit" (the magnitude of the move is defined by the learning rate) in the direction opposite to the gradient. The whole process is made possible by the fact that neural networks are chains of differentiable tensor operations, and thus it is possible to apply the chain rule of derivation to find the gradient function mapping the current parameters and current batch of data to a gradient value.

The loss is the quantity that you will attempt to minimize during training, so it should represent a measure of success on the task you are trying to solve. The optimizer specifies the exact way in which the gradient of the loss will be used to update parameters.

4 PROBLEM STATEMENT AND PROPOSED SOLUTIONS

In this chapter we will state the problem of the current project using the framework of supervised learning developed in the preceding chapters. We will also describe two methods to find the ground truth to control traffic light.

4.1 Problem statement

The goal is to build an adaptive traffic signal controller for an isolated intersection. The task of the controller is to reduce the total number of vehicles waiting in line before the intersection, keeping the balance between the conflicting directions. The controller should be adaptive, i.e. it should be constantly reacting to the changing non-stationary environment. The environment is the road junction. The proposed method fully connected neural network is suitable to be extended to a transport network of arbitrary size, and this is one of the major generalizations that we anticipate to make in the future.

4.2 State space

The state signal represents the aggregated number of queuing vehicles in non-conflicting directions. The

5    EXPERIMENTS, IMPLEMENTATION NOTES AND IMPROVEMENT PROSPECTS

CONCLUSION

6 REFERENCES

[1] J. Barceló, editor. Fundamentals of Traffic Simulation. Springer, 2010. DOI: 10.1007/978-1-4419-6142-6. 24, 53, 56

[2] E. Cascetta. Transportation Systems Analysis – Models and Applications, volume 29 of Springer Optimization and Its Applications. Springer, 2009. 27, 33, 38, 42

[3] R. P. Roess, E. S. Prassas, and W. R. McShane. Traffic Engineering. Prentice Hall, 3rd edition, 2004. 3, 10, 13, 16, 18, 21, 22, 45, 46

[4] Francois Chollet,2017. Deep learning with Python. Sixth edition, complete draft. The Manning Publications.

APPENDIX A

import os,sys

import traci

import subprocess

import time

from intersection_controller import IntersectionController

import utility

# generate different signal plans

signal_plans = []

for i in range(3):

for j in range(3):

sp = [20+i*20, 20+j*20]

signal_plans.append(sp)

#======================= INIT TRACI =======================

PORT = 8873

sumo_binary = 'sumo' if True else 'sumo-gui'

if 'SUMO_HOME' in os.environ:

tools = os.path.join(os.environ['SUMO_HOME'], 'tools')

sys.path.append(tools)

else:

sys.exit("please declare environment variable 'SUMO_HOME'")

if not os.path.exists('dataset'):

os.makedirs('dataset')

for fn in os.listdir("simulation_model/demand/jtrrouter1/rou_5/"):

if fn.endswith(".xml"):

for j,sp in enumerate(signal_plans):

print(fn,sp)

start = time.time()

sumo_process = subprocess.Popen([sumo_binary,

"--net-file", "simulation_model/netgen/two_intersections_v01.net.xml",

"--route-files","simulation_model/demand/jtrrouter1/rou_5/%s" %(fn),

"--additional-files","simulation_model/two_intersections_v01.add.xml",

"--begin","0",

"--end", "35000",

"--gui-settings-file","simulation_model/two_intersections_v01.settings.xml",

"--remote-port", str(PORT), "--no-step-log"], stdout=sys.stdout, stderr=sys.stderr)

traci.init(PORT)

node0 = IntersectionController('1_1', {'NS':['12_11'], 'SN':['10_11'], 'WE':['01_11'],'EW':['21_11']},{'NS':sp[0], 'WE':sp[1]}, 20)

node1 = IntersectionController('2_1', {'NS':['22_21'], 'SN':['20_21'], 'WE':['11_21'],'EW':['31_21']},{'NS':sp[0], 'WE':sp[1]}, 20)

node0.set_traffic_light_logic(sp[0],sp[1])

node1.set_traffic_light_logic(sp[0],sp[1])

step = 0

#print (sp)

while step < 35000:

#print ('\tSimulation step: %i' %(step))

node0.update_measures(step)

node1.update_measures(step)

traci.simulationStep()

step+=1

traci.close()

utility.report_measurements('dataset/measure'+fn.split('.')[0].replace('test','')+'_'+str(j)+'.csv', node0,node1)

print ('The simulation is complete!', 'Execution time:', time.time() - start)

sumo_process.wait()

APPENDIX B

import io

def report_measurements(out_file_name,node1, node2):

measurements1 = node1.measurements

measurements2 = node2.measurements

acq_time = [item[0] for item in measurements1]

ns1_cars = [item[1] for item in measurements1]

sn1_cars = [item[2] for item in measurements1]

we1_cars = [item[3] for item in measurements1]

ew1_cars = [item[4] for item in measurements1]

ns2_cars = [item[1] for item in measurements2]

sn2_cars = [item[2] for item in measurements2]

we2_cars = [item[3] for item in measurements2]

ew2_cars = [item[4] for item in measurements2]

ns1_delay = [item[5] for item in measurements1]

sn1_delay = [item[6] for item in measurements1]

we1_delay = [item[7] for item in measurements1]

ew1_delay = [item[8] for item in measurements1]

ns2_delay = [item[5] for item in measurements2]

sn2_delay = [item[6] for item in measurements2]

we2_delay = [item[7] for item in measurements2]

ew2_delay = [item[8] for item in measurements2]

gr_dur_ns1 = [item[9] for item in measurements1]

gr_dur_we1 = [item[10] for item in measurements1]

gr_dur_ns2 = [item[9] for item in measurements2]

gr_dur_we2 = [item[10] for item in measurements2]

cur_phase_id1 = [item[11] for item in measurements1]

cur_phase_id2 = [item[11] for item in measurements2]

remain_time = [item[12] for item in measurements1]

with io.open(out_file_name, 'w') as f:

f.write('TIME,NS1,SN1,WE1,EW1,NS2,SN2,WE2,EW2,NS1_DELAY,SN1_DELAY,WE1_DELAY,EW1_DELAY,NS2_DELAY,SN2_DELAY, WE2_DELAY,EW2_DELAY,gr_dur_ns1,gr_dur_we1,gr_dur_ns2,gr_dur_we2,PHASE_ID1,PHASE_ID2,REMAINING_TIME\n')

f.writelines(str(step)+ u',' +str(ns1)+ u',' + str(sn1)+ u',' + str(we1) + u',' + str(ew1)+ u','+str(ns2)+ u',' + str(sn2)+ u',' + str(we2) + u',' + str(ew2)+ u',' +str(ns1_d)+ u',' + str(sn1_d)+ u',' + str(we1_d) + u',' + str(ew1_d) + u','+str(ns2_d)+ u',' + str(sn2_d)+ u',' + str(we2_d) + u',' + str(ew2_d) + u',' + str(gr_ns1) + u','+ str(gr_we1) + u',' + str(gr_ns2) + u','+ str(gr_we2) + u',' + str(phId1) + u',' + str(phId2) + u',' + str(rTime) + u'\n' for (step,ns1,sn1,we1,ew1,ns2,sn2,we2,ew2,ns1_d,sn1_d,we1_d,ew1_d,ns2_d,sn2_d,we2_d,ew2_d,gr_ns1,gr_we1,gr_ns2,gr_we2,phId1,phId2,rTime) in zip(acq_time,ns1_cars,sn1_cars,we1_cars,ew1_cars,ns2_cars,sn2_cars,we2_cars,ew2_cars,ns1_delay,sn1_delay,we1_delay,ew1_delay,ns2_delay,sn2_delay,we2_delay,ew2_delay,gr_dur_ns1,gr_dur_we1,gr_dur_ns2,gr_dur_we2,cur_phase_id1,cur_phase_id2,remain_time))

APPENDIX C

from keras.models import Sequential

from keras.layers import Dense

from keras.initializers import RandomNormal

from keras.utils import np_utils

from keras.models import load_model

import numpy as np

import pandas as pd

import matplotlib.pyplot as plt

import io

def regression_model(in_dim, out_dim):

model = Sequential()

model.add(Dense(out_dim,kernel_initializer='normal',input_dim=in_dim))

model.compile(loss='mean_squared_error',  optimizer='adam', metrics=['mse'])

return model

def classification_model(in_dim, out_dim):

model = Sequential()

model.add(Dense(250,kernel_initializer='normal',input_dim=in_dim, activation='relu'))

model.add(Dense(250, kernel_initializer='normal',activation='relu'))

model.add(Dense(250,kernel_initializer='normal', activation='relu'))

model.add(Dense(out_dim,kernel_initializer='normal', activation='softmax'))

model.compile(loss='categorical_crossentropy',	optimizer='adam', metrics=['accuracy'])

return model

if __name__ == "__main__":

TRAIN = True

CLASSIFICATION = True

# load data

dataset = pd.read_csv("gt\ground_truth_joint.csv").values

X = dataset[:,:-5].astype(float)

print(X)

print(np.shape(X))

if TRAIN:

if CLASSIFICATION:

input_size = 8

output_size = 81

y = np_utils.to_categorical(dataset[:,-5].astype(int),num_classes=81)

print(np.shape(y))

print('d',dataset[:,-5])

print(np.shape(dataset[:,-5]))

model = classification_model(input_size,output_size)

model.save("model\cl_joint_model_%d_%d.h5" %(input_size,output_size))

else:

input_size = 8

output_size = 81

y = dataset[:,-4:]

print(y)

model = regression_model(input_size,output_size)

model.save("model\rg__joint_model_%d_%d.h5" %(input_size,output_size))

model.summary()

# Fit the model

model.fit(X, y, epochs=1500, batch_size=81)

model.save_weights("model\cl_joint_model_weights_%d_%d.h5"%(input_size,output_size))

else:

input_size = 8

output_size = 81

model = load_model("model\cl_joint_model_%d_%d.h5"%(input_size,output_size))

model.load_weights("model\cl_joint_model_weights_%d_%d.h5"%(input_size,output_size))

predictions = model.predict_classes(X[:3,:])

#predictions = model.predict(X[:3,:])

print(predictions)

print (dataset[:3,-5].astype(int))

history = model.fit(X, y, epochs=1500, batch_size=81)

train_acc = history.history['acc']

tr_loss = history.history['loss']

print(train_acc)

loss, acc = model.evaluate(X, y)

print("loss, acc ", loss, acc )

plt.figure(1)

plt.plot(train_acc)

plt.title('model accuracy')

plt.ylabel('accuracy')

plt.xlabel('epoch')

plt.legend(['train'], loc='upper left')

plt.show()

# summarize history for loss

plt.figure(2)

plt.plot(tr_loss)

plt.title('model loss')

plt.ylabel('loss')

plt.xlabel('epoch')

plt.legend(['train'], loc='upper left')

plt.show()

with io.open('model/report.csv', 'a') as f:

f.write(str(train_acc)+ u',' + str(acc)+ u','  + str(loss) + u'\n')

APPENDIX D

import os

import numpy as np

import pandas as pd

import matplotlib.pyplot as plt

import time

path_to_demands = "simulation_model/demand/jtrrouter1/rou_5/"

path_to_measures = "dataset/"

RESULT_DIR = "res/"

if not os.path.exists(RESULT_DIR):

os.makedirs(RESULT_DIR)

os.makedirs(RESULT_DIR+'stats_1/')

os.makedirs(RESULT_DIR+'stats_2/')

os.makedirs(RESULT_DIR+'stats/')

os.makedirs(RESULT_DIR+'plots/')

def estimate_ground_truth(fnames,path):

"""

This function sorts the measures w.r.t. the increasing queue and delay;

then it prints out the measures corresponding to the best signal plan

"""

my_data = pd.DataFrame()

for fname in fnames:

#print (fname)

df = pd.read_csv(os.path.join(path, fname))

df = df.set_index('TIME')

temp = df[['NS1','SN1','WE1','EW1','NS2','SN2','WE2','EW2','NS1_DELAY','SN1_DELAY','WE1_DELAY','EW1_DELAY','NS2_DELAY','SN2_DELAY',' WE2_DELAY','EW2_DELAY']].mean().to_frame().T

temp['FILE'] = fname.split('.')[0]

#temp['FILE1'] = fname.split('.')[0]

#temp['FILE2'] = fname.split('.')[0]

my_data = pd.concat([my_data,temp])

#print(my_data)

my_data['AVG_QUEUE'] = my_data[['NS1','SN1','WE1','EW1','NS2','SN2','WE2','EW2']].mean(axis=1)

my_data['AVG_DELAY'] = my_data[['NS1_DELAY','SN1_DELAY','WE1_DELAY','EW1_DELAY', 'NS2_DELAY','SN2_DELAY',' WE2_DELAY','EW2_DELAY']].mean(axis=1)

my_data['AVG_QUEUE_NS1'] = my_data[['NS1','SN1']].mean(axis=1)

my_data['AVG_QUEUE_WE1'] = my_data[['WE1','EW1']].mean(axis=1)

my_data['AVG_DELAY_NS1'] = my_data[['NS1_DELAY','SN1_DELAY']].mean(axis=1)

my_data['AVG_DELAY_WE1'] = my_data[['WE1_DELAY','EW1_DELAY']].mean(axis=1)

my_data['AVG_QUEUE1'] = my_data[['NS1','SN1','WE1','EW1']].mean(axis=1)

my_data['AVG_DELAY1'] = my_data[['NS1_DELAY','SN1_DELAY','WE1_DELAY','EW1_DELAY']].mean(axis=1)

my_data['AVG_QUEUE_NS2'] = my_data[['NS2','SN2']].mean(axis=1)

my_data['AVG_QUEUE_WE2'] = my_data[['WE2','EW2']].mean(axis=1)

my_data['AVG_DELAY_NS2'] = my_data[['NS2_DELAY','SN2_DELAY']].mean(axis=1)

my_data['AVG_DELAY_WE2'] = my_data[[' WE2_DELAY','EW2_DELAY']].mean(axis=1)

my_data['AVG_QUEUE2'] = my_data[['NS2','SN2','WE2','EW2']].mean(axis=1)

my_data['AVG_DELAY2'] = my_data[['NS2_DELAY','SN2_DELAY',' WE2_DELAY','EW2_DELAY']].mean(axis=1)

# sequential sort: first, sort w.r.t. AVG_QUEUE, then w.r.t. AVG_DELAY, etc. for each intersection

my_data1 = my_data.sort_values(['AVG_QUEUE1', 'AVG_DELAY1','AVG_QUEUE_NS1','AVG_QUEUE_WE1','AVG_DELAY_NS1','AVG_DELAY_WE1'], ascending=[True, True,True, True,True, True])

my_data1 = my_data1.set_index('FILE')

my_data2 = my_data.sort_values(['AVG_QUEUE2', 'AVG_DELAY2','AVG_QUEUE_NS2','AVG_QUEUE_WE2','AVG_DELAY_NS2','AVG_DELAY_WE2'], ascending=[True, True,True, True,True, True])

my_data2 = my_data2.set_index('FILE')

# sequential sort: first, sort w.r.t. OBJ_FUNCTION, then w.r.t. AVG_QUEUE1 and AVG_QUEUE2, etc. for two intersections

#1x12 matrix with values 1

#coefs = np.array([[2,2,2,2,1,1,1,1,1,1,1,1]])

coefs = np.array([[0.09,0.09,0.09,0.09,0.08,0.08,0.08,0.08,0.08,0.08,0.08,0.08]])

#vars = (12x9 matrix with values of the following values)

vars = my_data[['AVG_QUEUE1', 'AVG_QUEUE2','AVG_DELAY1', 'AVG_DELAY2',

'AVG_QUEUE_NS1', 'AVG_QUEUE_NS2','AVG_QUEUE_WE1', 'AVG_QUEUE_WE2',

'AVG_DELAY_NS1','AVG_DELAY_NS2','AVG_DELAY_WE1', 'AVG_DELAY_WE2']].values.transpose()

#obj_function = (9x1 matrix linear combination of all variables with coefficients 1)

obj_function = (np.matmul(coefs, vars)).transpose()

#print(obj_function)

my_data_united = my_data.insert(0,'OBJ_FUNCTION', obj_function)

my_data_united = my_data.sort_values(['OBJ_FUNCTION', 'AVG_QUEUE1', 'AVG_QUEUE2','AVG_DELAY1', 'AVG_DELAY2','AVG_QUEUE_NS1', 'AVG_QUEUE_NS2','AVG_QUEUE_WE1', 'AVG_QUEUE_WE2','AVG_DELAY_NS1','AVG_DELAY_NS2','AVG_DELAY_WE1', 'AVG_DELAY_WE2'], ascending=[True, True,True,True, True,True, True, True, True,True, True,True, True])

#my_data_united = my_data.sort_values(['OBJ_FUNCTION'], ascending=[True])

my_data_united = my_data_united.set_index('FILE')

fname = fnames[0][:fnames[0].rfind('_')]

my_data_united.to_csv(RESULT_DIR+'stats/res_'+fname+'.csv', sep=',')

my_data1.to_csv(RESULT_DIR+'stats_1/res_'+fname+'.csv', sep=',')

my_data2.to_csv(RESULT_DIR+'stats_2/res_'+fname+'.csv', sep=',')

# return the first five indeces (file names)

my_data1[['AVG_QUEUE_NS1','AVG_QUEUE_WE1','AVG_DELAY_NS1','AVG_DELAY_WE1','AVG_QUEUE1', 'AVG_DELAY1']].head().index.tolist()

my_data2[['AVG_QUEUE_NS2','AVG_QUEUE_WE2','AVG_DELAY_NS2','AVG_DELAY_WE2','AVG_QUEUE2', 'AVG_DELAY2']].head().index.tolist()

return my_data_united[['AVG_QUEUE_NS1','AVG_QUEUE_WE1','AVG_DELAY_NS1','AVG_DELAY_WE1','AVG_QUEUE1',

'AVG_DELAY1', 'AVG_QUEUE_NS2','AVG_QUEUE_WE2','AVG_DELAY_NS2','AVG_DELAY_WE2','AVG_QUEUE2', 'AVG_DELAY2']].head().index.tolist(), my_data1[['AVG_QUEUE_NS1','AVG_QUEUE_WE1','AVG_DELAY_NS1','AVG_DELAY_WE1','AVG_QUEUE1', 'AVG_DELAY1']].head().index.tolist(), my_data2[['AVG_QUEUE_NS2','AVG_QUEUE_WE2','AVG_DELAY_NS2','AVG_DELAY_WE2','AVG_QUEUE2', 'AVG_DELAY2']].head().index.tolist()

if __name__ == "__main__":

# get list of demand files

demands =  os.listdir(path_to_demands)

demands = [file.split('.')[0] for file in demands if file.endswith(".rou.xml")]

my_str_united = "PROB_NS1,PROB_SN1,PROB_WE1,PROB_EW1,PROB_NS2,PROB_SN2,PROB_WE2,PROB_EW2,SIGNAL_PLAN_INDEX,GR_NS1,GR_WE1,GR_NS2,GR_WE2\n"

my_str = "PROB_NS1,PROB_SN1,PROB_WE1,PROB_EW1,PROB_NS2,PROB_SN2,PROB_WE2,PROB_EW2,SIGNAL_PLAN_INDEX1,SIGNAL_PLAN_INDEX2,GR_NS1,GR_WE1,GR_NS2,GR_WE2\n"

start = time.time()

# separately estimate the ground truth signal plan

# for each demand

for dem in demands:

print (dem)

fnames = os.listdir(path_to_measures)

# take only those measures that correspond to a specific demand

measures = [measure for measure in fnames if ('_').join(dem.split('_')[1:]) in measure]

# get first five records

best_measures_united, best_measures_1, best_measures_2 = estimate_ground_truth(measures, path_to_measures)

#plot_data(path_to_measures,best_measures)

gt_united = best_measures_united[0].split('_')[1:]

gt_1 = best_measures_1[0].split('_')[1:]

gt_2 = best_measures_2[0].split('_')[1:]

#print(gt_united, gt_1, gt_2)

# 20 sec is the difference between adjucent signal plans

# 20 sec is the smallest phase duration

# 3 is the count of variances from 20 to 60 sec

gt_united_NS1 = int(gt_united[-1])//3*20+20

gt_united_WE1 = int(gt_united[-1])%3*20+20

gt_united_NS2 = int(gt_united[-1])//3*20+20

gt_united_WE2 = int(gt_united[-1])%3*20+20

#print(gt_united_NS1, gt_united_WE1, gt_united_NS2, gt_united_WE2)

#these are for joint experiment

gt_NS1 = int(gt_1[-1])//3*20+20

gt_WE1 = int(gt_1[-1])%3*20+20

gt_NS2 = int(gt_2[-1])//3*20+20

gt_WE2 = int(gt_2[-1])%3*20+20

#print(gt_NS1, gt_WE1, gt_NS2, gt_WE2)

# write to file the best signal plan

my_str_united+=((',').join(gt_united)+','+str(gt_united_NS1)+','+str(gt_united_WE1)+','+str(gt_united_NS2)+','+str(gt_united_WE2)+'\n')

#print(my_str_united)

my_str+=((',').join(gt_1)+','+gt_2[-1]+','+str(gt_NS1)+','+str(gt_WE1)+','+str(gt_NS2)+','+str(gt_WE2)+'\n')

#print(my_str)

print ('The ground truth is ready!', 'Execution time:', time.time() - start)

exec = time.time() - start

with open("ground_truth_joint1.csv", "w") as f:

f.write(my_str_united)

with open("ground_truth_independent1.csv", "w") as f:

f.write(my_str)